# ODEM Control Protocol

Contents

1.	Overview	2

2.	Protocol Architecture	2

3.	General Response Format	2

3.1.	Success Response Structure	2

3.2.	Error Response Structure	2

Error Codes	3

3.3.	Progress Response Structure	3

4.	Protocol Command	4

4.1.	Command 1: Write (32-bit values)	4

4.2.	Command 2: Read	4

4.3.	Command 3: Generic Control Commands	5

Control IDs	5

4.4.	Command 4: Optotune Commands	5

Command 4: Sub-Command IDs	5

Command 4 Error Handling	6

4.5.	Command 5: MSGDMA Streaming Commands	8

4.6.	Command 6	8

4.7.	Command 7: I2C Write (Extended Format)	8

4.8.	Command 8: I2C Read (Extended Format)	9

4.9.	Command 9: Ping/Echo (Connection Test)	10

4.10.	Command 10: SPI Register Write	10

4.11.	Command 11: AWG DAC Vector Transfer	11

4.12.	Command 12	11

4.13.	Command 13: Optotune Set Frequency (DEPRECATED)	12

# Overview

This protocol provides TCP-based communication for controlling the ODEM (Optical Distance Measurement) embedded Linux application.

The application runs on ARM Cortex-A9 with Intel Arria 10 FPGA hardware and provides real-time control of LIDAR data streaming, AWG DAC operations, Optotune mirror control, and I2C device management.

# Protocol Architecture

The protocol is organized with logical command separation:

- Linux endian functions for cross-platform compatibility

- Enum-based command system for maintainability and type safety

- Progress reporting system for long-running operations with real-time status updates

- Dedicated error codes for specific failure conditions (e.g., pattern files not found)

- MSGDMA commands under dedicated Command 5 for data streaming control

- Optotune commands under Command 4 with comprehensive sub-command structure

Add messages data is Big-endian.

# General Response Format

All responses from the server, whether indicating success or an error, begin with a unified header structure designed for 32-bit alignment.

## Success Response Structure

| Offset | Size | Field | Description |
| --- | --- | --- | --- |
| 0 | 1 | Response Type | `0x00` (Indicates Success) |
| 1 | 1 | Original Command ID | The command ID from the original request |
| 2 | 2 | Reserved | 0x00, 0x00` (For alignment and future use) |
| 4 | 4 | Length | Payload length |
| 8 | Length | Payload | The command-specific success data |

Size in bytes

## Error Response Structure

| Offset | Size | Field | Description |
| --- | --- | --- | --- |
| 0 | 1 | Response Type | `0x01` (Indicates Error) |
| 1 | 1 | Original Command ID | The command ID from the original request |
| 2 | 2 | Reserved | 0x00, 0x00` (For alignment and future use) |
| 4 | 4 | Error Message Length | Length of the Error Message String |
| 8 | 4 | Error Code | A specific numeric code |
| 12 | Variable | Error Message | ASCII string detailing the error. Not null-terminated |

Size in bytes

### Error Codes

| Error Code | Description |
| --- | --- |
| 0x00000000 | Operation completed successfully |
| 0x00000001 | General/unspecified error (used for directory creation failures, SPI errors, etc.) |
| 0x00000002 | Unknown or unsupported command |
| 0x00000003 | Invalid command parameters |
| 0x00000004 | Failed to open I2C device |
| 0x00000005 | I2C transaction failed |
| 0x00000006 | Physical memory access failed |
| 0x00000007 | Invalid command ID |
| 0x00000008 | Pattern files not found (Optotune Run with Pattern sub-command 9) |
| 0x00000009 | Optotune hardware operation failed (sub-commands 1, 2, 3, 5) |
| 0x0000000A | Optotune data logging operation failed (sub-commands 4, 9) |

## Progress Response Structure

Progress messages use a 16-byte header followed by a 4-byte message-length field and the ASCII status string:

| Offset | Size | Field | Description |
| --- | --- | --- | --- |
| 0 | 1 | Response Type | `0x02` (Indicates Progress Update) |
| 1 | 1 | Original Command ID | The command ID from the original request |
| 2 | 2 | Reserved | 0x00, 0x00` (For alignment and future use) |
| 4 | 4 | Payload Length | Size of the bytes **after** the header (message length field + status message) |
| 8 | 4 | Current Step | Current progress step (1-based) |
| 12 | 4 | Total Steps | Total number of steps in the operation |
| 16 | 4 | Status Length | Status message length (not null-terminated) |
| 12 | Variable | Status Message | ASCII status string |

Size in bytes

Progress Response Usage:

- Sent during long-running operations (Optotune calibration, logging operations, MSGDMA control)

- Multiple progress responses may be emitted for a single command

- Always followed by a final Success or Error response when the operation completes

- Clients should parse the `Status Length` field after reading the 16-byte header, then read the indicated number of bytes for the message

- `Current Step` and `Total Steps` provide completion percentage: `(Current Step / Total Steps) * 100`

# Protocol Command

The supported commands are:

- Command 1: Write (32-bit values)

- Command 2: Read

- Command 3: Generic Control commands

- Command 4: Optotune commands

- Command 5: MSGDMA streaming commands

- Command 6: Reserved

- Command 7: I2C write

- Command 8: I2C read

- Command 9: Ping/Echo (Connection Test)

- Command 10: SPI Register Write

- Command 11: AWG DAC Vector Transfer

- Command 12: Reserved

- Command 13: (Deprecated

## Command 1: Write (32-bit values)

  - Purpose: Write 32-bit data to a specified address or register.

  - Format:

    - Byte 0: Command (1)

    - Byte 1: Index/ID (typically 0)

    - Bytes 2-5: Address (32-bit, big-endian)

    - Bytes 6-9: Length (number of 32-bit values, big-endian)

    - Bytes 10+: Data payload (32-bit values, big-endian)

  - Total packet length: 10 bytes header + (length × 4) bytes data

  - Response:

    - The response is prefixed by the 8-byte **Success Response Header**.

    - Following the header:

    - Data payload: (length × 4) bytes of 32-bit values (big-endian)

  - Usage: 
Used to write configuration or control values to the FPGA or peripherals.

## Command 2: Read

  - Purpose: Read 32-bit data from a specified address or register.

  - Format:

    - Byte 0: Command (2)

    - Byte 1: Index/ID (typically 0)

    - Bytes 2-5: Address (32-bit, big-endian)

    - Bytes 6-9: Length (number of 32-bit values to read, big-endian)

  - No data payload in request

  - Total packet length: 10 bytes header only

  - Response:

    - The response is prefixed by the 8-byte **Success Response Header**.

    - Following the header:

    - Data payload: (length × 4) bytes of 32-bit values (big-endian)

  - Usage: 
Used to read status or configuration from the FPGA or peripherals.

## Command 3: Generic Control Commands

  - Purpose: Issue basic control operations (reset).

  - Format:

    - Byte 0: Command (3)

    - Byte 1: Control code/ID (see table below)

    - Bytes 2-5: Address (typically 0, reserved)

    - Bytes 6-9: Length (typically 0, reserved)

  - No data payload

  - Total packet length: 10 bytes header only

  - Usage: 
Used for basic system control operations.

### Control IDs

| ID | Name | Description | Response |
| --- | --- | --- | --- |
| 1 | Reset MCU | Stop sampling, exit sending task, and reset MCU | Success response with no payload |
| 2 | Get Version | Get application version information | Success response with null-terminated version string as payload |

The version format is:

- major.minor (timestamp), e.g. "0.97 (2025-10-21 14:30:45)"

- The timestamp reflects the build time from CMakeLists.txt

- Payload length includes the null terminator

## Command 4: Optotune Commands

  - Purpose: Control Optotune mirror operations.

  - Format:

    - Byte 0: Command (4)

    - Byte 1: Sub-command ID (see table below)

    - Bytes 2-5: Address (typically 0, reserved)

    - Bytes 6-9: Length (payload length for commands that require parameters)

    - Optional payload for certain commands

  - Total packet length: 10 bytes header + optional payload

  - Usage: 
Used for Optotune mirror control and logging operations.

### Command 4: Sub-Command IDs

| ID | Name | Description | Payload Required | Progress Reporting |
| --- | --- | --- | --- | --- |
| 1 | Run | Start Optotune operation with optional mode parameter | Optional (1 byte mode) | 5 steps |
| 2 | Run with Calibration | Start Optotune operation with calibration | No | 7 steps (setup + 5 iterations + completion) |
| 3 | Stop | Stop Optotune operation | No | No |
| 4 | Acquire Log | Acquire Optotune logger data | No | No |
| 5 | Run with Adaptive Calibration | Start Optotune with adaptive calibration | No | Variable steps |
| 6 | SPI Logging Control | Control SPI logging to file | 1 byte action | No |
| 7 | SPI Register Read | Direct read of Optotune SPI register | 2 bytes: system_id, register_id | No |
| 8 | SPI Register Write | Direct write to Optotune SPI register | 6 bytes: system_id, register_id, value | No |
| 9 | Run with Pattern | Start Optotune with existing pattern files | pattern identifier string | 5 steps with file paths |
| 10 | Reserved | Reserved for future use | No | No |
| 11 | Write LUT from Last Log | Write LOCATION_LUT using last measured log data | Optional (8 bytes: uint32_t optotune_points, uint32_t fpga_points) | No |
| 12 | Set VPU and Logger Frequencies | Set both VPU and Logger frequencies | 8 bytes: 2 IEEE 754 floats | No |
| 13 | Get VPU and Logger Frequencies | Read current VPU and Logger frequencies | No | No |

### Command 4 Error Handling

| Description | Sub-commands | Error Code |
| --- | --- | --- |
| Optotune Hardware Failures | 1, 2, 3, 5 | 0x00000009 |
| Optotune Logging Failures | 4, 9 | 0x0000000A |
| Pattern Files Not Found | 9 | 0x00000008 |
| General Failures (1) | Any | 0x00000001 |

(1) Each error type provides specific error messages for better client handling:

- Pattern files not found: "Pattern files not found for the specified identifier"

- Hardware failures: "Optotune [operation] failed", "Failed to open Optotune device"

- Logging failures: "Optotune data logging operation failed", "Optotune log acquisition failed"

- Directory creation failures: "Failed to create pattern results directory"

Optotune Commands Parameters

| Name | Parameters | Response |
| --- | --- | --- |
| Run mode (1) | - 0: Use preset files - 1: Generate test pattern (default if no payload provided) |  |
| SPI Logging Control (6) | - 0: Stop SPI logging - 1: Start SPI logging - 2: Get SPI logging status |  |
| SPI Register Read (7) | - Byte 0: System ID (0x00-0xFF) - Optotune internal system identifier - Byte 1: Register ID (0x00-0xFF) - Register address within the system | Success response with 4-byte payload containing the 32-bit register value |
| SPI Register Write (8) | - Byte 0: System ID (0x00-0xFF) - Optotune internal system identifier - Byte 1: Register ID (0x00-0xFF) - Register address within the system - Bytes 2-5: Value to write to the register (32-bit, big-endian) | Success response with no payload |
| Run with Pattern Parameter (9) | Null-terminated string identifying the pattern files to use (1-255 bytes) (1) | Success response with no payload |
| Write LUT from Last Log (11) (2) | Optional 8 bytes (if not provided, both parameters default to 0 for auto-calculation) - Bytes 0-3: OptoTune points (32-bit unsigned integer, big-endian) - Number of OptoTune logger samples to use (0 = auto-calculate) - Bytes 4-7: FPGA points (32-bit unsigned integer, big-endian) - Number of FPGA sample points for LUT (0 = auto-calculate) | Success response with no payload |
| Set VPU and Logger Frequencies (12) | - Bytes 0-3: VPU frequency (32-bit IEEE 754 float, big-endian) - Hz - Bytes 4-7: Logger frequency (32-bit IEEE 754 float, big-endian) - Hz | Success response with no payload |
| Get VPU and Logger Frequencies (13) | None | - Bytes 0-3: Current VPU frequency (32-bit IEEE 754 float, big-endian) - Hz - Bytes 4-7: Current Logger frequency (32-bit IEEE 754 float, big-endian) - Hz |

Notes:

- Pattern File Naming Conventions:
1. {pattern_id}_waveformX.csv` and {pattern_id}_waveformY.csv
2. waveformX_{pattern_id}.csv and waveformY_{pattern_id}.csv
3. pattern{pattern_id}_X.csv and pattern{pattern_id}_Y.csv

- Prerequisites: Must have previously run Optotune Acquire Log (sub-command 4) or other logging operation that generates `measured_xp.csv` and `measured_yp.csv` files

## Command 5: MSGDMA Streaming Commands

  - Purpose: Control MSGDMA (Memory-Mapped to Streaming DMA) operations.

  - Format:

    - Byte 0: Command (5)

    - Byte 1: Sub-command ID (see table below)

    - Bytes 2-5: Address (typically 0, reserved)

    - Bytes 6-9: Length (typically 0, reserved)

  - No data payload

  - Response:
Success response with no payload for MSGDMA commands.

  - Total packet length: 10 bytes header only

  - Usage: 
Used for high-speed data streaming control.

Command 5 Sub-Command IDs

| ID | Name | Description |
| --- | --- | --- |
| 1 | Start MSGDMA Streaming | Start MSGDMA continuous streaming with hardware device control |
| 2 | Stop MSGDMA Streaming | Stop MSGDMA continuous streaming and retrieve performance statistics |

Usage Notes:

- Sub-command 1 performs device availability check before starting streaming

- Sub-command 2 returns detailed performance statistics in the response

- Requires proper system configuration for hardware access

## Command 6

## Command 7: I2C Write (Extended Format)

  - Purpose: Write data to an I²C device, supporting muxes, register addressing, and variable data widths.

  - Packet Format:

| Offset | Size | Field | Description |
| --- | --- | --- | --- |
| 0 | 1 | Command code | 0x07 |
| 1 | 1 | Bus | I²C bus number on the host |
| 2 | 1 | Mux address | I²C-mux upstream address |
| 3 | 1 | Mux channel | Downstream channel number |
| 4 | 1 | Device address | 7-bit I²C slave address |
| 5 | 1 | Options | See SPI options byte details below |
| 6 | 4 | Length (M) | Number of data words to follow (big-endian) |
| 10 | 0-2 | Register addr | 0, 1, or 2 bytes, as specified by Reg-Addr-Width |
| … |  | M×(W/8) ¦ Data payload | M × (data-width/8) bytes of data |

  - Total packet length: 10 header bytes + Reg-Addr-Width + M × (data-width/8)

## Command 8: I2C Read (Extended Format)

  - Purpose: Read data from an I²C device, supporting muxes, register addressing, and variable data widths.

  - Packet Format:

| Offset | Size | Field | Description |
| --- | --- | --- | --- |
| 0 | 1 | Command code | 0x08 |
| 1 | 1 | Bus | I²C bus number on the host |
| 2 | 1 | Mux address | I²C-mux upstream address |
| 3 | 1 | Mux channel | Downstream channel number |
| 4 | 1 | Device address | 7-bit I²C slave address |
| 5 | 1 | Options | See options byte details below |
| 6 | 4 | Length (N) | Number of bytes to read (big-endian) |
| 10 | 0-2 | Register addr | 0, 1, or 2 bytes, as specified by Reg-Addr-Width |

  - Total packet length: 10 header bytes + Reg-Addr-Width; then host reads back N bytes.

SPI Options Byte Details

- Reg-Addr-Width (bits 5–4):

  - 00 = none

  - 01 = 8-bit pointer

  - 10 = 16-bit pointer

  - 11 = reserved

- Data-Width Code (bits 3–2):

  - 00 = 8-bit data

  - 01 = 16-bit data

  - 10, 11 = reserved

- Speed Code (bits 1–0):

  - 00 = Standard-mode ≤ 100 kbit/s

  - 01 = Fast-mode ≤ 400 kbit/s

  - 10, 11 = reserved

- This uses 6 bits for your three fields and leaves bits 7–6 free for future flags (e.g. 10-bit addressing or PEC).

SPI Example Packets

- DAC63202 (Channel 3, I²C 0x4B) Write DAC-X-DATA 0x19

07 00 70 03 4B 14 00 00 00 01 19 FF F0

  - 07 = CMD_I2C_WRITE

  - 00 = I²C bus 0

  - 70 = MUX address

  - 03 = MUX channel

  - 4B = device address

  - 14 = Options: Reg-Addr-Width=01 (8 bit) · Data-Width=01 (16 bit) · Speed=00 (100 kHz)

  - 00 00 00 01 = length = 1 word (2 bytes)

  - 19 = register = DAC-1-DATA (I²C addr 0x19)

  - FF F0 = 12-bit full-scale, left-aligned in 16 bits

- ADS7828 (Channel 4, I²C 0x48) Write pointer to start CH2 conversion (command = 0xC2)

07 00 70 04 48 10 00 00 00 00 C2

  - 10 = Options: Reg-Addr=01·Data-Width=00 (8 bit)·Speed=00

  - 00 00 00 00 = length 0 → no post-payload

  - C2 = command/pointer byte

- Read conversion result (1×16-bit word)

08 00 70 04 48 04 00 00 00 01

  - 04 = Options: Reg-Addr=00 (Don’t write any register after the address)·Data-Width=01 (16 bit)·Speed=00

  - 00 00 00 01 = length 1 word

  - → slave returns 2 bytes: MSB (D11–D4), then LSB (D3–D0 padded)

## Command 9: Ping/Echo (Connection Test)

  - Purpose: Simple connectivity test to verify the TCP connection is alive.

  - Format:

    - Byte 0: Command (9)

    - Byte 1: ID (unused, can be 0)

    - Bytes 2-5: Address (unused, can be 0)

    - Bytes 6-9: Field Length (unused, can be 0)

  - Response:

    - The response is prefixed by the 8-byte Success Response Header

  - Payload Length in the header will be 0.

  - There is no data payload following the header for a Ping response.

  - Usage:

    - Used by clients to periodically test connection health

    - Can be sent at regular intervals to keep TCP connection alive

    - Provides a simple way to measure round-trip time

    - No side effects on system operation

## Command 10: SPI Register Write

  - Purpose: Write a single 32-bit value to an SPI register with configurable effective bits.

  - Format:

    - Byte 0: Command (10)

    - Byte 1: Sub-device Address (0-255, used as SPI sub-device identifier)

    - Bytes 2-5: Address (reserved, set to 0)

    - Bytes 6-9: Field Length (number of effective bits: 0-22, where 0 means 22 bits, big-endian)

    - Bytes 10-13: Value (32-bit value to write, big-endian)

  - Total packet length: 14 bytes (10 header + 4 payload)

  - Response:

    - The response is prefixed by the 8-byte **Success Response Header**.

  - Payload Length in the header will be 0.

  - There is no data payload following the header for a successful write.

  - Usage:

    - Used to configure SPI-connected devices (e.g., DACs, ADCs)

    - The `Field Length` parameter specifies how many bits of the 32-bit value are significant

    - Value 0 in `Field Length` defaults to 22 bits (common for many SPI devices)

    - The implementation validates that the value fits within the specified bit range

  - Error Conditions:

    - Returns ERROR_CODE_GENERAL_ERROR if Field Length > 22

    - Returns ERROR_CODE_GENERAL_ERROR if Value exceeds the maximum for the specified number of bits

    - Returns ERROR_CODE_GENERAL_ERROR if SPI communication fails

  - Implementation Details:

    - Maximum effective bits: 22 (defined by DRIVERS_FPGA_MAX_SPI_EFFECTIVE_BITS)

    - Value validation: For N bits, maximum value is (2^N - 1)

    - Calls internal function spi_write_register_32() for actual SPI communication

## Command 11: AWG DAC Vector Transfer

  - Purpose: Transfer a vector of DAC values to the AWG (Arbitrary Waveform Generator) via SPI.

  - Format:

    - Byte 0: Command (11)

    - Byte 1: Sub-device Address (0-7, DAC sub-device identifier)

    - Bytes 2-5: Reserved (set to 0)

    - Bytes 6-9: Field Length (number of 16-bit DAC values, big-endian)

    - Bytes 10+: DAC Values (field_length × 2 bytes of 16-bit DAC values, big-endian)

  - Total packet length: 10 bytes header + (field_length × 2) bytes data

  - Response:

    - The response is prefixed by the 8-byte Success Response Header

    - Payload Length in the header will be 0.

    - There is no data payload following the header for a successful transfer.

  - Usage:

    - Used to upload waveform data to DAC devices for arbitrary waveform generation

    - Each DAC value is 16 bits (allowing for high-resolution waveforms)

    - Sub-device address allows targeting specific DAC channels (0-7)

  - Error Conditions:

    - Returns GENERAL_ERROR if SPI communication fails

    - Returns GENERAL_ERROR if sub-device address is invalid

  - Implementation Details:

    - Calls internal function `spi_send_dac_vector()` for actual SPI communication

    - Supports vectorized transfer for efficient bulk data upload

    - Big-endian byte order for both header fields and DAC values

## Command 12

## Command 13: Optotune Set Frequency (DEPRECATED)

⚠️ DEPRECATED:
This command is deprecated as of protocol version 2.1. Please use Command 4, Sub-commands 12 and 13 instead for frequency control with read capability.